Title

Effect of Microencapsulation of Bacteria and Fungi on PAH Biodegradation Success

Authors

Erica Babusci, Amelia Foley, Joshua Crittenden and Claudia Gunsch

Abstract

EPA Superfund sites are characterized as areas contaminated with hazardous levels of constituents, where remediation is necessary to protect environmental and human health. Polyaromatic hydrocarbons (PAHs) are a group of chemicals often found in Superfund sites that have mutagenic and carcinogenic properties. Remediating PAHs can be difficult due to their persistence and accumulative nature within the environment. There are various methods to remediate PAH contamination but many of those approaches are environmentally intrusive. Bioremediation is one treatment approach that is minimally invasive and cost-effective, especially compared to other physico-chemical treatment strategies. However, preceding in-situ bioremediation applications have shown inconsistent PAH degradation because planktonic microbes cannot readily establish in a novel environment. In this project, we aim to develop a strategy to encapsulate microbes to improve the establishment success, viability, and fitness of bacteria and fungi for in-situ bioremediation. Currently, we have identified a library of PAH-degrading bacterial and fungal strains, including bacteria of the genera Novosphingobium, Alcaligenes, Sphingobium, Hydrogenphaga, and Stenotrophomonas and fungi of the genera Alternaria, Arthrinium, Cladosporium, Didymella, Fusarium, Paraphaeosphaeria, Pyrenochaetopsis, Sarocladium, Septoriella, and Trichoderma. Microcapsules are being generated via extrusion using three biodegradable polymers (alginate, alginate with chitosan coat, and polyvinyl alcohol). Methods quantifying viability and growth for each biopolymer are still being developed. Current methods are comparing planktonic to encapsulated microbial growth in a defined medium to investigate which biopolymer best supports microbial growth. These data are essential for determining proper methods of encapsulation to promote the success of bioremediation in PAH contaminated sites.