Biotechnology, Microbiology, and the Built Environment

Title

Metagenomic Analysis of Microbial Genera on Solar Panels

Presenting Author

Erica Babusci (erica.babusci@duke.edu)

Department of Civil and Environmental Engineering, Duke University

Abstract

Solar panels are a renewable energy source that help reduce dependence on fossil fuels by converting sunlight into power through photovoltaic cells. However, their efficiency can be significantly hindered by the accumulation of environmental particles on their surfaces, which obstruct light absorption and lead to energy and revenue losses. While particles such as pollen and dust were traditionally believed to be the primary contributors to this efficiency loss, recent evidence suggests that microbial activity plays a substantial role. Microbial biofilms and fungal filaments appear to exacerbate the accumulation of particles by creating “sticky” surfaces that not only trap environmental debris but also make solar panels more difficult to clean. This growth is hypothesized to serve as catalysts for further accumulation, accelerating efficiency degradation over time. To address this issue, we are collaborating with Solar Unsoiled to investigate the microbial communities present on solar panels at three solar farms across North Carolina. Swab samples collected from panel surfaces were processed and analyzed using a PromethION sequencer, which provides high-quality, high-throughput metagenomic data. This cutting-edge sequencing technology enabled us to identify microbial taxa at the genus level, revealing both bacterial and fungal communities. Preliminary results show that bacterial genera including Hymenobacter, Deinococcus, Methylobacterium, Sphingomonas, Roseomonas, and Spirosoma, and fungal genera including Aspergillus, Trebouxia, Alternaria, Colletotrichum, Exophiala, and Aureobasidium are the most abundant. These genera have been shown to possess traits such as resistance to drought, heat, and radiation, which enhance their survival and enable them to endure the environmental stressors commonly associated with solar panels. Additionally, many of these genera are known for their ability to form biofilms, likely contributing to their persistence on solar panels and the accumulation of environmental particles. The next step involves conducting a meta-transcriptomic analysis on the samples to identify the metabolic pathways involved in biofilm formation. Understanding the microbial composition and the functioning pathways will enable the development of targeted strategies to mitigate microbial growth on solar panels, thereby enhancing their efficiency.

Abstract Guidelines

– Title of abstract: 150 characters or less.
– Abstract word limit: 350 words maximum.